How to upgrade DMR chip?

1. Turn radio off

2. Connect to PC

3. Start HF685X Software upgrade tools

4. click Browse to insert bin file, select com port and speed, click Burn

5. While the device is still turned off, hold the menu button while turning it on to start the upgrade process. It should start automatically.

Note: In case you do all the steps and progress bar is at 0% and timer is running, stop everything and repeat all steps but change the "speed" to other than 1Mbps.