Update Notes – 2025.03.21

- Added Promiscuous Mode for each digital channel. When enabled, the channel can receive calls even if the color code does not match.

- Added Channel ID feature for each channel:

  - Set the Channel ID via: Channel Set -> Channel ID (16).

  - Choose to use either the device ID or the Channel ID during transmission via: Channel Set -> Channel ID Set (17).

- Added multiple tail tone options under Channel Set -> Tail Tone for analog channels:

  - 55Hz

  - 120° phase shift

  - 180° phase shift

  - 240° phase shift

- Updated the memory driver to support 25Q256 and 25Q512 flash chips (default factory configuration remains 25Q128, users may replace the chip themselves if needed).

- Added SSB (Single Side Band) reception option for analog channel receiving.

- Expanded the digital UHF frequency range to 400-520MHz.

- Updated the sub-audio display on the main screen for analog channels:

  - Changed from displaying "CTC" or "DCS" to showing the actual sub-audio code, such as 67.0 or D023N.

- Hold the "0" key while powering on, and after 3 seconds, the radio will restore the programmed data.

- This function can be used to recover data in case of data loss (indicated by both the red and green LEDs lighting up simultaneously).