To fix the rt-4d data lost

STEP1: use CPS software to recovery it

- Insert programming cable into PC and radio, power on radio

- Select COM port,

- insert the factory default frequency

- use super password 707070 to remove old password

- press WRITE to write data into radio

If above method can’t fix the issue then try with step 2

STEP2: use radio tuning software

Open DMR radio tuning software and select COM port and press Write to write data into radio (don’t change any other data)