2025-4-23

Adding beacon function, you can set beacon duration and intervals from CPS

CPS: MD7500_CPS_V1.0.0.106

Firmware:

main_tx_V03.02.01.006_u

main_tx_V03.02.01.006_v

2024-9-2

to add RDAC (Repeater diagnostics and control) function, please kindly note that after updating, repeater need to adjust the power at factory, so we didn't suggest you to update it by end user, it will be operated by professionals

Firmware version:

VHF:repeater_main_tx_v_v3.8

UHF:repeater_main_tx_u_v3.8

Programming software: RepeaterRdac_V1.0.0.4

WAN, it needs to be fix IP

LAN，IP under one router can be done

If you want to diagnose repeater not in the same local network, you should get trap port and IP and this IP should be fixed one

If you want to diagnose repeater in the same local network, IP under one router can be done

2024-4-27: to add following functions

1. to add mix channel into repeater

2. to update display more professional

3. to add ARC4, AES128 and AES256

4. some repeaters are installed in high area outside and if they are defective, communications will be affected, so we add one function to connect two repeaters together and one is normally working and one is dormant, two repeaters are connected from backside DB26 connector via special cable from TYT , if master repeater is off suddenly or RX unlock or TX unlock, dormant repeater will be activated to work. To use this function, please buy that special cable from us together with repeater.

5. to send call sign automatically in certain interval time in CWID

6. to add two functions to DB26 pins, one pin to detect tone code and the other pin to detect carrier

PL/DPL (CTCSS/DCS) dedect(analog) or Color Code Dedect(digital)

When repeater receive the correct CTCSS/DCS or color code, this pin will be 0V. When it does not receive correct CTCSS/DCS on the signal or it didnt receive any CTCSS/DCS or color code , it will be 5V)

Carrier Dedect (COS/COR

Carrier dedection (COS)

when carrier is detected, it will be 0V, when carrier is not detected, it will be 5V or 3.3V

Updating firmware operation

- please install USB driver(CH341SER) first to PC

- To keep repeater to be on, and connect it to PC via programming cable, and open the updated software: MD-7500_DWN, firstly, please choose correct comm. port and open comm. port, secondly, please open the latest firmware, and click update, when it shows download waiting, please turn off the repeater and turn on the repeater again at once, repeater will be in updating now, which will take 7 minutes, after updating, please reboot the repeater.

- the CPS in the updated software is to download the default programming file into repeater, if data is missing due to wrong firmware or wrong programming, it will cause repeater not power on, please write data into repeater again, the operation is the same as firmware updating, To keep repeater to be on, and connect it to PC via programming cable, and open the updated software: MD-7500_DWN, firstly, please choose correct comm. port and open comm. port, secondly, please open the programming file and click update. after updating, please reboot the repeater.

- the logo in the updated software is to download the power-on logo into repeater, before download photo, you should change the photo format, open , and click file/open to open one file resolution as 160*128, bit depth as 24, color photo and please save as dta file, lastly, please rename the file as bin file, and to download into radio, to keep repeater to be off, and connect it to PC via programming cable, and open the updated software: MD-7500_DWN, firstly, please choose correct comm. port and open comm. port, secondly please choose the bin file you saved in the PC and click update